|  |  | ALBERTA FOREST PRODUCTS ASSOCIATION 900, 10707 100th Avenue EDMONTON, ALBERTA T5J 3M1 PHONE: (780) 452-2841 FAX: (780) 455-0505 |
| --- | --- | --- |

A.F.P.A. FALL 2012 GRADE EXAM

Name:		____________________________________	Date:	______________________

Company:	____________________________________	Location: ____________________

Mark the answer you choose with an [X]. ONLY ONE ANSWER PER QUESTION PLEASE!

| 1. | An indication that a tree will be categorized as a Hardwood is: |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Needles | [ ] Branches | [ ] Leaves | [ ] Knots |

| 2. | Which part of the trunk has no active function? |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Cambium | [ ] Sapwood | [ ] Bast | [ ] Heartwood |

| 3. | The healing agent in the tree is called: |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Bark | [ ] Cambium | [ ] Pith | [ ] Pitch |

| 4. | A SKIP 1/16 inch deep full length, would be classified as? |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Hit or Miss | [ ] Heavy Skip | [ ] Hit and Miss | [ ] Very Heavy |

| 5. | A Grade Stamp must be what % readable: |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] 50% | [ ] 75% | [ ] 95% | [ ] 100% |

| 6. | A growth ring of, one band of Springwood and one band of Summerwood is called: |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Tree ring | [ ] Circle ring | [ ] One annual ring | [ ]Two annual rings |

| 7. | A deviation EDGEWISE from a line drawn END to END, is what kind of warp? |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Bow | [ ] Crook | [ ] Cup | [ ] Twist |

| 8. | Lumber grade stamped or sold as KD-HT or S-DRY must NOT exceed the moisture content of: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | [ ] 15% | [ ] 19% | [ ] 23% | [ ] 26% |  |  |  |

| 9. | A SHAKE that extends from one surface of a piece to an adjoining surface is called: |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Standard | [ ] Surface | [ ] Through | [ ] 3 Face Shake |

| Mark the following DEFECTS under the correct CLASSIFICATION: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Defect | Natural | Manufacturing | Seasoning |
| 10. | Spike Knot | [ ] | [ ] | [ ] |
| 11. | Ring shake | [ ] | [ ] | [ ] |
| 12. | Skip | [ ] | [ ] | [ ] |
| 13. | Cup | [ ] | [ ] | [ ] |

| Match the following MANUFACTURING DEFECTS to the correct DEPTH CLASSIFICATION: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DEFECT |  | 1/64" | 1/32" |  | 1/16" |  | 1/8" | Over 1/8" |
| 14. | Medium Skip |  | [ ] | [ ] |  | [ ] |  | [ ] | [ ] |
| 54. | Heavy Loosened Grain |  | [ ] | [ ] |  | [ ] |  | [ ] | [ ] |
| 16. | Very Light Offset |  | [ ] | [ ] |  | [ ] |  | [ ] | [ ] |
| 17. | Light Mismatch |  | [ ] | [ ] |  | [ ] |  | [ ] | [ ] |
| Mark the correct dressed WIDTH for the following: |  |  |  |  |  |  |  |  |  |
| 18. | 2 x 4 S-GRN | [ ] 3 1/2" |  |  | [ ] 4" |  | [ ] 3 9/16" |  | [ ] 3 5/8" |
| 19. | 2 x 6 S-GRN HT | [ ] 5 1/2" |  |  | [ ] 5 5/8" |  | [ ] 6” |  | [ ] 5 9/16" |
| 20. | 2 x 8 KD-HT | [ ] 7 9/16" |  |  | [ ] 7 1/2" |  | [ ] 7 5/8 " |  | [ ] 7 1/4" |
| 21. | 2 x 10 S-DRY | [ ] 9 1/4" |  |  | [ ] 9 3/4" |  | [ ] 9 1/2" |  | [ ] 10" |

| Mark the correct dressed THICKNESS for the following: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 22. | 2 x 10 S-GRN HT | [ ] 1 3/4" | [ ] 1 9/16" | [ ] 1 5/8"" | [ ] 1 1/2" |
| 23. | 2 x 10 KD-HT | [ ] 1 7/16" | [ ] 1 1/2" | [ ] 1 9/16" | [ ] 1 5/8" |
| 24. | 3 x 6 KD-HT | [ ] 2 1/2 | [ ] 2 9/16 | [ ] 3 | [ ] 2 1/4 |
| 25. | 1 x 10 S-GRN | [ ] 3/4" | [ ] 25/32" | [ ] 15/16" | [ ] 1" |

| Mark the Highest grade of Par.121 Stud that allows the following for a 2 x 6 - 104 5/8” long: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DEFECT |  |  |  |  |  |  |  | STUD |  |  |  | ECONOMY |  |  |  |
| 26. |  | Wane 2/3 thickness on the end |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 27. |  | 100% soft White Speck |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 28. |  | Wane 1/2 width full length |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 29. |  | Skip 1/16” full length on one edge |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 30. |  | Spike Knot 50% displacement |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 31. |  | Split 1/6 length |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| Mark the HIGHEST grade of Par. 122 Light Framing that allows the following for a 2 x 4 - 16 feet long: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DEFECT |  |  |  | ## CONSTRUCTION |  |  |  | ## STANDARD |  |  | UTILITY |  |  | ## ECONOMY |  |  |
| 32. | Wane 1/2 T X 1/3 W full length |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 33. | Surface Shake 4 feet long |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 34. | Full length wane, 1/4 thickness |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 35. | Decay 1/12 the width |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 36. | White Speck 50% of the volume |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 37. | Split 33 inches long |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 38. | Wane completely through narrow face away from the end |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 39. | Light warp |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 40. | Firm honeycomb 1/6 the width |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| Mark the HIGHEST grade of Par. 124 Structural Light Framing and Joist & Planks that allows the following in a 2 x 6-10 feet long: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DEFECT |  |  | SEL STR |  | #1 |  |  | #2 |  | #3 |  |  | NONE |  |
| 41. |  |  | White Speck 1/2 the volume |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 42. |  |  | Wane on END 1/2 width |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 43. |  |  | Wane 2/3 thickness on the END |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 44. |  |  | Slope of Grain 1 in 8 |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 45. |  |  | 20% Heart Stain |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 46. |  |  | Honeycomb 1/6 the width |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 47. |  |  | Skip 1/16 inch deep full length |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 48. |  |  | Through Shake 5 feet long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 49. |  |  | Wane 1/4 T x 1/4 W full length |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 50. |  |  | Light Warp |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 51. |  |  | Decay 6 inches long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 52. |  |  | A saw cut |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 53. |  |  | Split 1/6 the length long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 54. |  |  | Wane completely across Wide Face 1 feet long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 55. |  |  | Surface Shake 10’L |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 56. |  |  | 240 Grub Holes on one face |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |

List the knot sizes for the following KD-HT 2 x 8’s

2 7/8"

57.	List the knot size:

SIZE

3 3/8"

3 5/8 "

58.	List the knot size:

3/4"

1-3/4”

SIZE

3"                  1 1/4"

59.	List the knot size

3/4"									  3/4"

SIZE

3 5/8”                                 3 5/8"

60.	List the knot size

1/2”                                                                                                     1/2"

3/4”	SIZE