|  |  | ALBERTA FOREST PRODUCTS ASSOCIATION 500, 10709 Jasper Avenue EDMONTON, ALBERTA T5J 3N3 PHONE: (780) 452-2841 FAX: (780) 455-0505 |
| --- | --- | --- |

A.F.P.A. SPRING 2010 GRADE EXAM

Name:		____________________________________	Date:	______________________

Company:	____________________________________	Location: ____________________

Mark the answer you choose with an [X]. ONLY ONE ANSWER PER QUESTION PLEASE!

| 1. | Which of the following species is a SOFTWOOD ? |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Aspen | [ ] Birch | [ ] Black Spruce | [ ] Cottonwood |

| 2. | Which of the following species is in the SPF group: |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Tamarack | [ ] Jack Pine | [ ] Douglas Fir | [ ] White Pine |

| 3. | Where is PITCH manufactured in a tree? |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Bark | [ ] Sapwood | [ ] Resin Ducts | [ ] Summerwood |

| 4. | Which type of wood cell, helps limit the shrinking and swelling of wood? |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Wood Rays | [ ] Pits | [ ] Cambium | [ ] Bast |

| 5. | Bark or lack of wood, on the edge or corner of a piece of lumber, is known as: |  |  |  |
| --- | --- | --- | --- | --- |
|  | [ ] Skip | [ ] Wane | [ ] Peck | [ ] Eased Edge |

6-9.		Match the characteristics with the description. (e.g.     a    )

a)	72” long

6.	1/3 the length of an 8’		_________

b)	24” long

c)	28” long					7.	1/4  the length of a 7’		__________

d)	21” long

8.	1/6 the length of a 10		__________

e)	32” long

f)	20” long					9.	1/2  the length of a 12’	__________

| Mark the following DEFECTS under the correct CLASSIFICATION: |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Defect | Natural | Manufacturing | Seasoning |
| 10. | Checks | [ ] | [ ] | [ ] |
| 11. | Warp | [ ] | [ ] | [ ] |
| 12. | Skip | [ ] | [ ] | [ ] |

| Match the following MANUFACTURING DEFECTS to the correct CLASSIFICATION: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DEFECT | Very light | Light | Medium | Heavy | Very Heavy |
| 13. | 1/16 inch Offset | [ ] | [ ] | [ ] | [ ] | [ ] |
| 14. | 1/32 inch Skip | [ ] | [ ] | [ ] | [ ] | [ ] |
| 15. | 1/4 inch Machine Bite | [ ] | [ ] | [ ] | [ ] | [ ] |

| Mark the correct dressed WIDTH for the following: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 16. | 2x6 S-GRN | [ ] 7 1/2" | [ ] 6" | [ ] 5 9/16" | [ ] 5 5/8" |
| 17. | 3x10 KD-HT | [ ] 9 1/2" | [ ] 10" | [ ] 9 1/4” | [ ] 9 1/8" |
| 18. | 2x12 S-GRN | [ ] 11 9/16" | [ ] 11 1/2" | [ ] 11 1/8 " | [ ] 11 1/4" |
| 19. | 2x4 S-DRY | [ ] 3 1/4" | [ ] 3 3/4" | [ ] 3 1/2" | [ ] 3 7/8" |

| Mark the correct dressed THICKNESS for the following: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 20. | 2x4 S-GRN HT | [ ] 1 3/4" | [ ] 1 9/16" | [ ] 1 1/2" | [ ] 15/8" |
| 21. | 2x6 KD-HT | [ ] 1 7/16" | [ ] 1 1/2" | [ ] 1 9/16" | [ ] 1 5/8" |
| 22 | 4x4 S-DRY | [ ] 3 1/2 | [ ] 3 9/16 | [ ] 4 | [ ] 3 1/4 |
| 23. | 3x10 KD-HT | [ ] 2 7/16" | [ ] 2 1/2" | [ ] 2 9/16" | [ ] 2 5/8" |

| Mark the Highest grade of Par.121 Stud that allows the following for a 2 x 6 - 92 5/8” long: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DEFECT |  |  |  |  |  |  |  | STUD |  |  |  | ECONOMY |  |  |  |
| 24. |  | Hit and Miss skip on NARROW FACE |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 25. |  | Decay 1/4 the cross section |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 26. |  | 1 inch of GOOD WOOD on NARROW FACE |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 27. |  | 1/2 inch of BOW |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 28. |  | A 3 inch EDGE Knot |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| 29. |  | A Split, 16 inches long |  |  |  |  |  |  |  | [ ] |  |  |  | [ ] |  |  |  |
| Mark the HIGHEST grade of Par. 122 Light Framing that allows the following for a 2 x 4 12 feet long: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DEFECT |  |  |  | ## CONSTRUCTION |  |  |  | ## STANDARD |  |  | UTILITY |  |  | ## ECONOMY |  |  |
| 30. | Hit or Miss Skip |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 31. | Saw Cut |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 32. | Full length Wane, 1/3 thickness |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 33. | Through Shake, 30 inches long |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 34. | 7/8 inch GOOD WOOD on END of WIDE FACE |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 35. | Firm White Speck |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 36. | Wane 1/2 the width, 16 inches long |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 37. | A 1 3/4 inch Knot Hole |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| 38. | A 1 3/4 inch Round Knot |  |  |  | [ ] |  |  |  | [ ] |  |  | [ ] |  |  | [ ] |  |  |
| Mark the HIGHEST grade of Par. 124 Structural Light Framing and Joist & Planks that allows the following in a 2 x 6-16 feet long: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DEFECT |  |  | SEL STR |  | #1 |  |  | #2 |  | #3 |  |  | NONE |  |
| 39. |  |  | White Speck 1/2 the volume |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 40. |  |  | Decay 1/2 the cross section |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 41. |  |  | Wane 7/8 thickness on the END |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 42. |  |  | Slope of Grain 1 in 12 |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 43. |  |  | 20 % Heart Stain |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 44. |  |  | A 1 3/4 inch EDGE Knot |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 45. |  |  | Skip 1/16 inch deep full length |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 46. |  |  | Through Shake 2 feet long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 47. |  |  | 1/2 inch Good Wood on Narrow Face, on the END |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 48. |  |  | Light Warp |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 49. |  |  | Wane 1/4 the Width Full Length |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 50. |  |  | A 4 1/2 inch Centerline Knot |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 51. |  |  | Split 12 inches long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 52. |  |  | Wane completely across Wide Face 2 feet long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 53. |  |  | 24 Grub Holes |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 54. |  |  | Saw Cut completely through the thickness, 7/8 inch deepest penetration |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 55. |  |  | Wane 1/2 the Width |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |
| 56. |  |  | Shake completely through Narrow Face (3 face) 4 feet long |  |  | [ ] |  | [ ] |  |  | [ ] |  | [ ] |  |  | [ ] |  |

List the knot sizes for the following KD-HT 2 x 4’s

1 3/8"

57.	List the knot size:

1 7/8"

SIZE

1 5/8"

58.	List the knot size:

3/8"

1-3/4”

1"                  1/2"

# SIZE

59.	List the knot size

1/2"                                                                                                      1”

1 3/4”                                 1 3/4"                  SIZE

60.	List the knot size

1/2"

3/4”

SIZE